ACCIDENT DETECTION WITH TIME DISPLAY

/*

* Fall Detection and Alert System

* Components: MPU6050, DS3231 RTC, GPS NEO-6M, GSM SIM900A

*

* Pin Connections:

* MPU6050 & DS3231: SDA->A4, SCL->A5

* GPS: TX->D4, RX->D3

* GSM: TX->D2, RX->D5

* Red LED: D7, Green LED: D8, Buzzer: D9

*/

#include <Wire.h>

#include <MPU6050.h>

#include <RTClib.h>

#include <SoftwareSerial.h>

// Pin Definitions

#define GPS_RX 4

#define GPS_TX 3

#define GSM_RX 2

#define GSM_TX 5

#define RED_LED 7

#define GREEN_LED 8

#define BUZZER 9

// Thresholds for fall detection

#define FALL_THRESHOLD 2.5      // G-force threshold

#define IMPACT_THRESHOLD 3.0    // High impact detection

#define STATIONARY_TIME 2000    // Time in ms to confirm fall (no movement)

// Emergency contact (update with actual number)

const String EMERGENCY_PHONE = "+1234567890";

// Object Initialization

MPU6050 mpu;

RTC_DS3231 rtc;

SoftwareSerial gpsSerial(GPS_RX, GPS_TX);

SoftwareSerial gsmSerial(GSM_RX, GSM_TX);

// Variables

float ax, ay, az;           // Accelerometer values

float totalAccel;           // Total acceleration magnitude

bool fallDetected = false;

unsigned long fallTime = 0;

String gpsData = "";

String latitude = "";

String longitude = "";

bool gpsFixed = false;

void setup() {

Serial.begin(9600);

Wire.begin();

gpsSerial.begin(9600);

gsmSerial.begin(9600);

// Pin modes

pinMode(RED_LED, OUTPUT);

pinMode(GREEN_LED, OUTPUT);

pinMode(BUZZER, OUTPUT);

// Initial LED state

digitalWrite(RED_LED, LOW);

digitalWrite(GREEN_LED, LOW);

digitalWrite(BUZZER, LOW);

Serial.println(F("Initializing Fall Detection System..."));

// Initialize MPU6050

if (!mpu.begin(MPU6050_SCALE_2000DPS, MPU6050_RANGE_2G)) {

Serial.println(F("MPU6050 initialization failed!"));

blinkError();

while (1);

}

mpu.calibrateGyro();

mpu.setThreshold(3);

Serial.println(F("MPU6050 initialized"));

// Initialize RTC

if (!rtc.begin()) {

Serial.println(F("RTC not found!"));

blinkError();

while (1);

}

if (rtc.lostPower()) {

Serial.println(F("RTC lost power, setting time..."));

rtc.adjust(DateTime(F(__DATE__), F(__TIME__)));

}

Serial.println(F("RTC initialized"));

// Initialize GSM

initGSM();

// Success indication

digitalWrite(GREEN_LED, HIGH);

delay(1000);

digitalWrite(GREEN_LED, LOW);

Serial.println(F("System Ready!"));

Serial.println(F("Monitoring for falls..."));

}

void loop() {

// Read accelerometer data

Vector rawAccel = mpu.readRawAccel();

Vector normAccel = mpu.readNormalizeAccel();

ax = normAccel.XAxis;

ay = normAccel.YAxis;

az = normAccel.ZAxis;

// Calculate total acceleration magnitude

totalAccel = sqrt(ax*ax + ay*ay + az*az);

// Read GPS data

readGPS();

// Fall detection algorithm

if (!fallDetected) {

// Check for sudden acceleration change (fall/impact)

if (totalAccel > IMPACT_THRESHOLD || totalAccel < 0.5) {

Serial.println(F("Possible fall detected!"));

fallDetected = true;

fallTime = millis();

digitalWrite(RED_LED, HIGH);

tone(BUZZER, 1000, 200);

}

} else {

// Check if person is stationary after fall

if (millis() - fallTime > STATIONARY_TIME) {

if (totalAccel < 1.2) {  // Still not moving much

Serial.println(F("FALL CONFIRMED!"));

handleFallAlert();

fallDetected = false;

digitalWrite(RED_LED, LOW);

} else {

// False alarm - person moved/recovered

Serial.println(F("False alarm - movement detected"));

fallDetected = false;

digitalWrite(RED_LED, LOW);

digitalWrite(GREEN_LED, HIGH);

delay(500);

digitalWrite(GREEN_LED, LOW);

}

}

}

// Debug output

if (millis() % 2000 < 50) {

Serial.print(F("Accel: "));

Serial.print(totalAccel);

Serial.print(F(" G | GPS: "));

Serial.println(gpsFixed ? "Fixed" : "Searching...");

}

delay(50);  // Sample rate ~20Hz

}

void readGPS() {

while (gpsSerial.available()) {

char c = gpsSerial.read();

gpsData += c;

if (c == '\n') {

if (gpsData.startsWith("$GPGGA")) {

parseGPGGA(gpsData);

}

gpsData = "";

}

}

}

void parseGPGGA(String nmea) {

int comma[15];

int count = 0;

for (int i = 0; i < nmea.length(); i++) {

if (nmea.charAt(i) == ',') {

comma[count++] = i;

}

}

if (count < 6) return;

// Check GPS fix quality

String fixQuality = nmea.substring(comma[5] + 1, comma[6]);

if (fixQuality == "1" || fixQuality == "2") {

gpsFixed = true;

// Extract latitude

String lat = nmea.substring(comma[1] + 1, comma[2]);

String latDir = nmea.substring(comma[2] + 1, comma[3]);

// Extract longitude

String lon = nmea.substring(comma[3] + 1, comma[4]);

String lonDir = nmea.substring(comma[4] + 1, comma[5]);

if (lat.length() > 0 && lon.length() > 0) {

latitude = convertToDecimal(lat, latDir);

longitude = convertToDecimal(lon, lonDir);

}

} else {

gpsFixed = false;

}

}

String convertToDecimal(String coord, String dir) {

if (coord.length() < 4) return "0.0";

int dotPos = coord.indexOf('.');

String degrees = coord.substring(0, dotPos - 2);

String minutes = coord.substring(dotPos - 2);

float dec = degrees.toFloat() + minutes.toFloat() / 60.0;

if (dir == "S" || dir == "W") {

dec = -dec;

}

return String(dec, 6);

}

void handleFallAlert() {

// Sound alarm

for (int i = 0; i < 3; i++) {

tone(BUZZER, 2000, 500);

delay(600);

}

// Get current time

DateTime now = rtc.now();

// Build alert message

String message = "FALL ALERT! ";

message += "Time: ";

message += String(now.hour()) + ":" + String(now.minute()) + ":" + String(now.second());

message += " Date: ";

message += String(now.day()) + "/" + String(now.month()) + "/" + String(now.year());

if (gpsFixed) {

message += " Location: ";

message += "https://maps.google.com/?q=" + latitude + "," + longitude;

} else {

message += " GPS: Not Fixed";

}

Serial.println(F("Sending SMS alert..."));

Serial.println(message);

// Send SMS

sendSMS(EMERGENCY_PHONE, message);

// Make call (optional)

// makeCall(EMERGENCY_PHONE);

digitalWrite(GREEN_LED, HIGH);

delay(2000);

digitalWrite(GREEN_LED, LOW);

}

void initGSM() {

Serial.println(F("Initializing GSM..."));

delay(2000);

gsmSerial.println("AT");

waitForResponse(1000);

gsmSerial.println("AT+CMGF=1");  // Text mode

waitForResponse(1000);

gsmSerial.println("AT+CNMI=1,2,0,0,0");  // SMS notification

waitForResponse(1000);

Serial.println(F("GSM Ready"));

}

void sendSMS(String number, String message) {

gsmSerial.println("AT+CMGS=\"" + number + "\"");

delay(1000);

gsmSerial.print(message);

delay(100);

gsmSerial.write(26);  // Ctrl+Z to send

waitForResponse(5000);

}

void makeCall(String number) {

gsmSerial.println("ATD" + number + ";");

waitForResponse(5000);

delay(20000);  // Call duration

gsmSerial.println("ATH");  // Hang up

waitForResponse(1000);

}

void waitForResponse(unsigned long timeout) {

unsigned long start = millis();

while (millis() - start < timeout) {

if (gsmSerial.available()) {

String response = gsmSerial.readString();

Serial.print(response);

}

}

}

void blinkError() {

for (int i = 0; i < 5; i++) {

digitalWrite(RED_LED, HIGH);

tone(BUZZER, 500, 200);

delay(300);

digitalWrite(RED_LED, LOW);

delay(300);

}

}